; Radio Repeater configuration file (for use with app_rpt)

;

; K5TRA BBB    27 August 2014

[XXXXX]					; node stanza:   XXXXX is YOUR node number

rxchannel=Simpleusb/usbXXXXX		; defines interface type: Simpleusb, Usbradio, Voter, or Zap

duplex=2				; set duplex or half-duplex mode: 0, 1, 2, 3, or 4

;	0 = half duplex

;	1 = semi-half duplex (with telemetry & courtesy tones)

;	2 = full duplex

;	3 = full duplex without repeater audio

;	4 = full duplex without repeater audio during autopatch

;controlstates=controlstates		; control state stanza name

scheduler=scheduleXXXXX		; scheduler stanza name

morse=morseXXXXX			; morse stanza name

macro=macroXXXXX			; macro stanza name

functions=functionsXXXXX		; functions stanza name

phone_functions=functionsXXXXX	; phone functions stanza name

link_functions=functionsXXXXX		; link functions stanza name

telemetry=telemetry			; telemetry stanza name

wait_times=wait-times			; wait times stanza name

context =  radio				; dialing context

callerid = "Repeater" <0000000000>	; caller ID for phone

idrecording = |iK5TRA			; insert YOUR call

accountcode=RADIO			; account code (optional)

hangtime=30				; squelch tail hang time (in mS)

althangtime=50				; alternate squelch tail hang time (in mS)    (optional)

totime=300000				; transmit time-out time (in mS)

idtime=300000				; ID interval time (in mS)

politeid=60000				; time before ID interval end to try ID in tail

startup_macro=*934*915*329521	; telemetry off, enable scheduler, connect to Austin 900 HUB

inxlat=*,#,0123456789*#ABCD	; remap inbound DTMF strings: function preamble, end character, alpha-numeric character remapping

outxlat=*,#,0123456789*#ABCD	; remap outbound DTMF strings: function preamble, end character, alpha-numeric character remapping

propagate_dtmf=yes			; allow DTMF string to be sent over Allstar link

parrotmode=1				; playback transmitted audio after PTT unkey

parrottime=1000			; Delay time (in mS) before transmitted audio is repeated

telemdefault=2				; Set default telemetry status:

;	0 = off

;	1 = on

;	2 = timed on status before off

telemdynamic=1	; allow or disallow user change of telemetry ; status with COP

; command.:  0 = disallow; 1 = allow.

unlinkedct=ct1				; local off-link tone

remotect=ct3				; remote base connected tone

linkunkeyct=ct8				; remote on-link tone

nounkeyct=0				; do not eliminate courtesy tones

beaconing=0				; do not ID on preset time intervals without repeater activity

; *** Status Reporting ***

;

statpost_program=/usr/bin/wget,-q,--timeout=15,--tries=1,--output-document=/dev/null

statpost_url=http://stats.allstarlink.org/uhandler.php ; Status updates

[morse XXXXX]				; morse stanza: defines morse params across multiple nodes

speed=25				; morse code speed (WPM)

frequency=750				; morse telemetry tone frequency

amplitude=1500			; morse telemetry tone amplitude

idfrequency=440			; morse ID tone frequency

idamplitude=1500			; morse ID tone amplitude

[schedule XXXXX]			; scheduler stanza: defines macros to be executed at specified

; times using crontab syntax

79=59 * * * *				; telem off

11=05 00 * * *				; connect K5TRA 900 MHz HUB

11=05 04 * * *				; repeat at 4:05 AM

11=05 08 * * *				; repeat at 8:05 AM

11=05 12 * * *				; repeat at 12:05 PM

11=05 16 * * *				; repeat at 4:05 PM

11=05 20 * * *				; repeat at 8:05 PM

[functions XXXXX] 			; functions stanza: defines functions called by DTMF sequences

01=ilink,11				; permanently disconnected link:  called by *01node-number

03=ilink,13				; permanently connected link:  called by *03node-number

1=ilink,1				; disconnect specified link:  called by *1node-number

2=ilink,2				; monitor specified link:  called by *2node-number

3=ilink,3				; connect specified link (transceive):  called by *3node-number

4=ilink,4				; Enter command on specified link:  called by *4node-number

5=macro				; preamble character (5) to perform macro from macro stanza

70=ilink,5				; system status

71=ilink,6				; disconnect all links

72=ilink,7				; last node to key-up

73=ilink,15				; full system status (all links)

74=ilink,16				; reconnect links disconnected with "disconnect all links"

75=ilink,8				; connect specified link - local monitor only

80=status,1				; force ID (global)

81=status,2				; say time of day (global)

915=cop,15				; scheduler enable

916=cop,16				; scheduler disable

925=cop,25				; query last node un-keyed

933=cop,33				; local telemetry output enable

934=cop,34				; local telemetry output disable

935=cop,35				; local telemetry output on demand

850=ilink,6				; link disable

851=ilink,16				; link enable

950=cop,3				; repeater transmit disable

951=cop,2				; repeater transmit enable

960=status,3				; say software version (global)

971=cop,21				; enable Parrot Mode

970=cop,22				; disable Parrot Mode

972=cop,55				; Parrot once if Parrot mode is disabled

973=cop,23				; Parrot cleanup/flush

995=cmd, /usr/local/sbin/astres.sh	; restart Asterisk

996=cmd, /usr/local/sbin/shut.sh	; reboot PC:  requires shell script ‘shut.sh’

997=cmd, /usr/local/sbin/on_hub.sh	; connect Austin 900 HUB:  requires shell script ‘on_hub.sh’

[macro XXXXX]				; macro stanza: defines macros called by DTMF sequences

; macro functions are combinations of individual functions

; macros are called by *5macro-number#

11=*71*934*915*329521#	; disconnect all, telemetry disable, scheduler on, connect HUB

12=*934*129521*915#			; telemetry disable, disconnect HUB, scheduler on,

30=*429521 *530 #			; remotely call macro 30 on node 29521 (must be connected)

31=*429521 *531 #			; remotely call macro 31 on node 29521 (must be connected)

60=*429521 *860 #			; remotely call function 860 on node 29521 (must be connected)

61=*429521 *861 #			; remotely call function 861 on node 29521 (must be connected)

62=*429521 *862 # 			; remotely call function 862 on node 29521 (must be connected)

63=*429521 *863 #			; remotely call function 863 on node 29521 (must be connected)

64=*429521 *864 #			; remotely call function 864 on node 29521 (must be connected)

65=*429521 *865 #			; remotely call function 865 on node 29521 (must be connected)

70=*935*70#				; telemetry on demand, pole list of connected nodes

71=*935*71#				; telemetry on demand, disconnect all

79=*934#				; telemetry off

80=*429521 *850 #			; remotely call function 850 on node 29521 (must be connected)

81=*429521 *851 #			; remotely call function 851 on node 29521 (must be connected)

82=*329521 *934 #			; remotely call function 934 on node 29521 (must be connected)

83=*935*429521 *70 #			; telemetry on demand, remotely call function 70 on node

; 29521 (must be connected)

[telemetry]				; telemetry stanza:  defines courtesy tones

ct1=|t(350,0,50,2048)(500,0,50,2048)(660,0,50,2048)

ct2=|t(660,880,150,2048)

ct3=|t(440,0,150,2048)

ct4=|t(550,0,150,2048)

ct5=|t(660,0,150,2048)

ct6=|t(880,0,150,2048)

ct7=|t(660,440,150,2048)

ct8=|t(660,0,50,1500)(0,0,25,0)(880,0,50,1500)

remotetx=|t(1633,0,50,1500)(0,0,25,0)(1209,0,50,1500)

remotemon=|t(1209,0,50,2048)

cmdmode=|t(900,903,200,2048)

functcomplete=|t(1000,0,100,2048)(0,0,100,0)(1000,0,100,2048)

patchup=rpt/callproceeding		; override default patch-up message

patchdown=rpt/callterminated		; override default patch-down message

[wait-times]				; wait-times stanza:  defines time delay for telemetry events

telemwait=250				; delay before sending general audio telemetry

idwait=600				; delay before sending ID audio telemetry

unkeywait=150				; delay before sending courtesy tone audio telemetry

calltermwait=2000			; delay after autopatch disconnect before call ended telemetry

[nodes]					;  nodes stanza: defines nodes addresses for LOCAL connections

; this allows connections between nodes on the same network

; not registered with allstar link

; syntax:

; nodenumber = radio@fqdn1/nodenumber,fqdn2 (or NONE)

;	 fqdn1 is fully qualified domain name to look up on

; 	outgoing connections

;	 fqdn2 is fully qualified domain name to look up on

; 	incoming connections (usually the same as fqdn1

;	NONE can be used for fqdn2 if no forward DNS checking

;	is required. Use for LOCAL nodes.

XXXXX = radio@127.0.0.1/XXXXX,NONE	; XXXXX is YOUR node number

; nodes stanza should contain this (usually only this).

YYYYY = radio@192.168.0.43/YYYYY,NONE	; YYYYYY is a LOCAL/PRIVATE node number

ZZZZZ = radio@192.168.0.44/ZZZZZ,NONE	; ZZZZZ is a LOCAL/PRIVATE node number

#includeifexists custom/rpt.conf