; SimpleUSB configuration

;		 -- YOU MUST CHOOSE: USBRADIO OR SIMPLEUSB OR VOTER –

;		USBRADIO uses CPU processing for COS and CTCSS decode

;		If you have COS available from radio, SIMPLEUSB is a better choice (THIS FILE)

;		If you are using an RTCM, then you must choose VOTER

;

; K5TRA BBB    27 August 2014

[general]

[usbXXXXX]				; usb stanza:   XXXXX is YOUR node number

hdwtype=0				; USB sound fob such as DMK URI

rxboost=0				; 20 dB pad inserted in audio stream from receiver (=1, no pad)

txboost=0				; 20 dB pad inserted in audio stream to transmitter (=1, no pad)

carrierfrom=usb			; use external COS  (=usbinvert, low on signal), (=no, not used)

ctcssfrom=no				; use radio CTCSS decode (alternatively, =usb or =usbinvert)

txmixa=no				; URI ‘a’ output not used

txmixb=voice				; transmit audio on ‘b’ output from URI

invertptt=0				; PTT pull down to key  (=1, pull-up to key)

duplex=1				; full duplex  (=0, half duplex)

rxondelay=5				; after transmit, ignore COS for 5x(20 mS) = 100 mS

plfilter=yes				; apply PL filter to audio stream

; deemphasis=yes			; enable de-emphasis (when using discriminator audio)

eeprom=0				; no EPROM present in URI

#includeifexists custom/usbradio.conf